The College of New Jersey Confined Space Entry Permit

To report an emergency, use a radio to contact Campus Police or dial 911 from a campus phone.

This permit must be displayed at the work area.

| Date of Issue: | Starting Date/Time: |  | Permit Number: |
| --- | --- | --- | --- |
| Location: |  |  |  |
| Job Description: |  |  |  |
| Employee(s) Assigned for Entry: |  | Employee Assigned as Standby: |  |

| Expected Hazards |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Inert Gas |  | Electrical |  | Radiation |  | Other: |
|  | Flammable |  | Ignition Source |  | Cutting/Welding |  |  |

| Entry Safety Checklist |  |  | Air Monitoring Checklist |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Required (X or N/A) | Implement (Initial or N/A) | Monitored Continuously for Oxygen (O2), Carbon Monoxide (CO), Flammability (LFL/LEL), and Hydrogen Sulfide (H2S) # Acceptable Levels |  |  |  |  |  |  |  |  |  |  |
| Confined Space Entry Training | X |  | O2 | 19.5%-23.5% |  |  | CO |  |  |  | 0-50ppm |  |  |
| Buddy System | X |  | LFL/LEL | 0%-10% |  |  | H2S |  |  |  | 0-10ppm |  |  |
| Continuous Forced Ventilation |  |  | Instruments Used: | Date Calibrated |  |  |  | By Whom |  |  |  |  |  |
| Local Exhaust Ventilation |  |  | Lumidor Air Monitor |  |  |  |  |  |  |  |  |  |  |
| Continuous Air Monitoring (see right) | X |  | QRae |  |  |  |  |  |  |  |  |  |  |
| Piping Blanked/Disconnected |  |  | ## Air Test Results | O2% |  | CO% |  |  |  | LEL% |  | H2Sppm |  |
| Respirator-Type: |  |  | Pre-Ventilation Results |  |  |  |  |  |  |  |  |  |  |
| Lifeline/Safety Harness | X |  | Pre-Entry Results |  |  |  |  |  |  |  |  |  |  |
| Other Protective Equipment |  |  | After above testing is complete, continue testing and document readings below every ____________________ thereafter. |  |  |  |  |  |  |  |  |  |  |
| Ground Fault Circuit Interrupter |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Fire Permit |  |  | Date Time |  | O2% | COppm |  |  | LEL% |  |  |  | H2Sppm |
| Other Permit |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Assigned/briefed Employees on Hazards | X |  |  |  |  |  |  |  |  |  |  |  |  |
| & Procedures |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Electrical System(s) Locked Out |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Mechanical System(s) Locked Out |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other |  |  |  |  |  |  |  |  |  |  |  |  |  |

I understand that is it my responsibility to implement the requirements listed above prior to entry, and that this permit is only valid so long as the requirements are met.  Should the location of circumstances of the above operations change, I understand this permit is VOID, and I will notify the Office of Occupational Safety and Environmental Services (x2881) immediately!

I have checked the above described location, and the nature of the hazardous operation, and grant permission for the described work to be performed.